### SECTION 3.3	GLASS HANDLING & FOREIGN MATTER

# Glass, Ceramic and Hard Brittle Plastics Breakages

Any broken items must be effectively removed from the processing and storage environments.

When handling broken items all possible precautions should be taken to avoid personal injury.

- Stop production if bottle broken on-line.

- Isolate the affected area where the items breakage occurred.

- Inform all staff who are likely to enter the area and restrict them from entering the area.

- Using a brush and shovel sweep up item pieces from the affected area(s) and surrounding area(s), including equipment and all other items.

- Discard directly into a designated bin.  Note: There is a designated recycling bin for broken glass.

- Thoroughly inspect the affected area(s) to ensure that all small pieces and fragments of the broken items are removed.

- Carry out a thorough clean-down of the affected equipment and environment.

- Note:  Should a bottle break on the bottling line, the sufficient number of bottles before and after the broken bottle shall be removed and discarded to waste.

- Record breakage and post clean-down inspection on the Glass Breakage Report.

# Handling and Storage of Glass Bottles

- The bottles are received in secure packaging to prevent contamination and ensure they are not damaged during transportation and storage.

- The pallet is fork-lifted into the bottling Room.

- The staff shall manually remove the outer plastic covering from the top down, layer by layer as required.

- Staff shall touch only the neck or body of the bottles.  Staff shall NOT touch the top/lip of the bottles.

- This ensures the stack remains stable.  Should only a portion of the pallet be used the stack can be re-wrapped to protect the bottles from the environment.

- There are cardboard pallet stabiliser positioned between each layer of bottles.

Glass Breakage Record

| Date & Time | Location | Number of bottles | Post clean inspection sign |
| --- | --- | --- | --- |