Section 6.2		Distilled Spirits HACCP

6.2.1	Scope of Distilled Spirits HACCP

| #### Product | Examples: Vodka, Whiskey, Gin |
| --- | --- |
| Process | From receipt of raw materials to dispatch of packaged beer |

    - Intended Consumer and Product Requirements

| Products | Examples: Vodka, Whiskey, Gin |
| --- | --- |
| Intended consumer | General public – Adults (not intended for children and pregnant women) |
| Intended use of product | Direct consumption Culinary use |
| Regulatory Standards that must be met | Current Food Act Composition of beer as specified in Standard 2.7.5 of the Food Standards Code Permitted additives and level of use as specified in Schedule 1 of Standard 1.3.1 of the Food Standards Code Permitted processing aids and level of use as specified in Schedules 1 of Standards 1.3.3 of the Food Sstandards Code Labelling as specified in Standard 1.2.3 and 2.7.1 of the Food Standards Code |

    - Identitification of Hazards from Inputs

| #### Inputs | Description/Specificaton | Biological Hazard (B) | Chemical Hazard (C) | Physical Hazard (P) |
| --- | --- | --- | --- | --- |
| Clean water | Complies with the requirements defined in the Food Act 2014 | None | None | None |
| Yeast | Suitable for food use | None | None | None |
| Grain | Suitable for food use Sourced from a supplier that complies with regulations regarding the management of agrichemicals | Enteric pathogens (e.g. Salmonella spp.) | Residues of agricultural chemicals (e.g. pesticides, fungicides) 1 | None |
| Additives (e.g. preservatives, fining agents) | Food grade. Permitted for use in vodka, and used within limits specificed in standard 1.3.1 of the Food Standards Code. | None | Sulphite 2 Allergens form fining agents with animal protein deriviatives 3 | None |
| Processing aids (e.g. enzymes, gases) | Permitted for food use as specified in Standard 1.3.3 of the Food Standards Code. | None | None | None |
| Spices | Food grade | Bacterial pathogens (e.g. Salmonella spp., Bacilllus spp., Clostridium spp.) | None | None |
| New bottles, cans | Sterilised | None | None | None |
| Used (reuse) bottles, kegs | Company specification | Bacterial pathogen | Chemical residues (e.g. if the bottle/keg had been reused to contain chemicals) | Foreign matter (e.g. glass, metal) |
| Caps | Suitable for food use | None | None | None |
| Labels, metal foil, plastic cover, cases | Company specifications | None | None | None |

- Grains may contian residues of agricultural chemicals, but grain that are sourced form suppliers that complies with good agaricultural practices, including the application of agrichemicals in accordance with their label authorisation, are likely to meet the New Zealand (Maximum Residue Limits of Agricultural Compounds) Food Standards 2005 (No.2), as amended.  Thus, chemical residues were not consider any further in the hazard analysis above.

- Sulphite can induce asthma in susceptible individuals.

- Residues of fining agents with animal protein derivatives can cause allergic reaciton in susciptable individuals.

    - Process Flow Diagram for Distilled Spirits

| - Inputs | ## Process Step | Outputs |
| --- | --- | --- |
|  | 1. Purchasing |  |
| Raw Materials: - Grain - Yeast - Additives / preservatives - Packaging | 2. Receiving |  |
|  | 3. Storing |  |
| - Grains | 4. Milling |  |
| - Water - Yeast | 5. Brewing |  |
|  | 6. Fermentation |  |
| - Additives / preservatives | 7. Distilling |  |
| - Fining agent / filter | 8. Filtering | - Fines |
|  | 9. Packaging |  |
| - Packaging | 10. Storing |  |
|  | 11. Dispatch/Transportation | - Packaged vodka |

    - Process Flow Diagram for Vodka

| ###### Process | Inputs | Hazard reasonably likely to occur on or in the product at this step | Justification | Q1. Is there a controlmeasure(s) for the hazards at this step? If yes, identify the control measure1 and then answer Q2 | Q2. Is this step a CCP2? |
| --- | --- | --- | --- | --- | --- |
| 1. Purchasing | All food and product contact raw materials | See Section 6.2.3 | Hazard from raw materials not meeting regulatory requirements. | Yes – The Company operates an Approved Supplier Programme. | No |
| 2. Receiving | All food and product contact raw materials | B – Enteric pathogens | Refer to 6.2.3 | None | No |
| 3. Storing | All food and product contact raw materials | B – Enteric pathogens | Hazards from poor storage methods and unclear environment. | Yes – The NP L3 includes a raw materials standard and procedure. See Section 2.2.2 Storage of Raw Materials. | No |
| 4. Milling | Grains | B – Enteric pathogens | Hazards carried from previous step | None |  |
| 5. Brewing | Water Yeast | B – Enteric pathogens | Hazards carried from previous step | Yes – Boiling of mix in kettle will eliminate vegetative pathogens. | No |
| 6. Fermentation | Grains, Yeast Water | B – Enteric pathogens | Hazards carried from previous step | Yes – Fermentation will eliminate vegetative pathogens. | No |
| 7. Distilling | Fermented vodka |  |  |  |  |

| ###### Process | Inputs | Hazard reasonably likely to occur on or in the product at this step | Justification | Q1. Is there a controlmeasure(s) for the hazards at this step? If yes, identify the control measure1 and then answer Q2 | Q2. Is this step a CCP2? |
| --- | --- | --- | --- | --- | --- |
| 8. Filtering | Filter / finging agent | C – Allergens | Refer to 6.1.3 Intended Consumer and Product Requirements | No 3 |  |
| 8. Packaging | Glass bottles | P – Glass framents | Incorrect filler operation can results in chipping | Yes – correct equipment set-up, equipment maintenance, routine observation during filling, filtration of any affected vodka. | No |
| 9. Storing | Packaged vodka | None |  |  |  |
| 10. Dispatch and Transportation | Packaged vodka | None |  |  |  |

- The procedures for the contorl measures are required to be documented in the Naitonal Programme Level 3.

- A CCP is a step at which an identified hazard can be eliminated or reduced to an accpetable level.  Control at the CCP must be linked to the achievement of an established food safety outcome (i.e. product or process critieria).  A CCP must have a defined critical limit which is measurable and capable of being monitored on a real time basis so that immediate corrective aciton can be undertaken.

- The risk associated with the presence of potential allergens from residues of fining agents with animal protein derivates can be managed by providing a warning label on the product.