Section 6.3	Non-Alcoholic Beverages HACCP

6.3.1	Scope of Non-Alcoholic Beverages HACCP

| #### Products | Non-Alcoholic Beverages e.g. Ginger Beer, Lemonade |
| --- | --- |
| Process | From receipt of raw materials to dispatch of packaged beverage. |

    - Intended Consumer and Product Requirements

| Products | Non-Alcoholic Beverages e.g. Ginger Beer, Lemonade |
| --- | --- |
| Intended consumer | General public – Adults |
| Intended use of product | Direct consumption |
| Regulatory Standards that must be met | Current Food Act Composition as specified in Standard 2.6.2 Non-Alcoholic Beverages and Brewed Soft Drinks Permitted additives and level of use as specified in Schedule 1 of Standard 1.3.1 of the Food Standards Code Permitted processing aids and level of use as specified in Schedules 1 of Standards 1.3.3 of the Food Sstandards Code Labelling as specified in the Food Standards Code |

    - Intended Consumer and Product Requirements

| #### Inputs | Description/Specificaton | Biological Hazard (B) | Chemical Hazard (C) | Physical Hazard (P) |
| --- | --- | --- | --- | --- |
| Clean water | Complies with the requirements defined in clause 6.1 of the Wine Standards Management Plan. | None | None | None |
| Fresh or frozen fruit | Sourced from a supplier that complies with regulations regarding the management of agrichemicals1. No dropped fruit. | Enteric pathogen (e.g. Salmonella spp., E. coli spp., Cryptosporidium spp.)2 Patulin in apples3 | Residues of agricultural chemicals (e.g. pesticides, fungicides) 4 | None |
| Fruit juice or pulp | Sourced from a supplier that complies with the requirement of the Food Act. | Enteric pathogen (e.g. Salmonella spp., E. coli spp., Cryptosporidium spp.)2 Patulin in apples3 |  |  |
| Additives (e.g. preservatives) | Food grade. Permitted for food use as speicified in Standard 1.3.1 of the Food Standards Code. | None | Sulphite 5 | None |
| Processing aids (e.g. enzymes, gases) | Permitted for food use as specified in Standard 1.3.3 of the Food Standards Code. | None | None | None |
| Sugar | Food Grade | None | None | None |
| Ginger root, Spices | Food grade | Bacterial pathogens (e.g. Salmonella spp., Bacilllus spp., Clostridium spp.) | None | None |
| Gas | Food grade | None | None | None |

| #### Inputs | Description/Specificaton | Biological Hazard (B) | Chemical Hazard (C) | Physical Hazard (P) |
| --- | --- | --- | --- | --- |
| New bottles, cans | Sterilised | None | None | None |
| Used (reuse) bottles, kegs | Sanitised | Bacterial pathogen | Chemical residues (e.g. if the bottle/keg had been reused to contain chemicals) | Foreign matter (e.g. glass, metal) |
| New caps | Suitable for food use | None | None | None |
| Labels, metal foil, plastic cover, cases | Company specifications | None | None | None |

- Operators who receive fresh fruit must make reasonable enquiries to check that they are suitable for making into cider.  The application of agrichemicals to food crops in New Zealand must be in accordance with their bale authorisation.

- Salmonella spp., E.coli )157, and Crytosporidium spp. Have been implicated in outbreaks in the United States involving the consumptio of unpasteurised apple juice (Johnson et.al., 2006).  The presnece of these enteric paqthogens on fruit and in fruit juice has been attributed to some form  of faecal contaminaito from animals grazaing in orchards (Keller and Miller, 2006).

Some orchar operators I New Zealand allow animals to graze in their orchards, thus there is potential for faecal contamiation of applies that drop on the ground.  Exclusion of dropped applies will minimise the occurrence of these hazards on the fruit ad in the extraction fruit juice.

- Patulin is a mycotoxin that is produced primarily by certain species of Penicillum, Aspergillus, and Byssoclamys moulds hat may grow on a veriety of foods including fruit.  Patulin has been found to occur ain applies and pears with brown rot, and in apple juice (US FDS 2000).  Contamination of applie juice has ben attributed to contaminaiton with mould  on apples with surace famage.  Although there are no known published reports on the occurrrence of patulin in applie juice and apple products produced in New Zzealand, the potential for putulin contaminaito in apples and applie juice have been consiered in the hazard analysis.  Control for patulin are covered in the hazared analysis above.

- Frush fruit may contian residues of agricultural chemicals, but frutis that are sourced form suppliers that cojmplh with good agaricultural practices, including the application of agrichemicals I accordance with their label authorisation, are likely to meet the New Zealand (Maximum Residue Limits of Agricultural Compounds) Food Standards 2005 (No.2), as amended.  Thus, chemical residues were not sondered any furhter in the hazard analysis above.

- Sulphite can induce asthma in susceptible individuals.

    - Process Flow Diagram for Non-Alcoholic Beverages

| - Inputs | ## Process Step | Outputs |
| --- | --- | --- |
|  | 1. Purchasing |  |
| Raw Materials: - Fruit - Sugar, spice - Additives / preservatives - Packaging | 2. Receiving |  |
|  | 3. Storing of Raw Material |  |
| - Sugar, water, fruit juice, spices | 4. Make |  |
| - Carbon dioxide | 5. Carbonation |  |
| - Packaging | 6. Packaging |  |
|  | 7. Storing |  |
|  | 8. Dispatch/Transportation | - Packaged beverage |

    - Process Flow Diagram for Non-Alcoholic Beverages

| ###### Process | Inputs | Hazard reasonably likely to occur on or in the product at this step | Justification | Q1. Is there a controlmeasure(s) for the hazards at this step? If yes, identify the control measure1 and then answer Q2 | Q2. Is this step a CCP? |
| --- | --- | --- | --- | --- | --- |
| 1. Purchasing | All food and product contact raw materials | See Section 6.3.3 | Hazard from raw materials not meeting regulatory requirements. | Yes – The Company operates an Approved Supplier Programme. | No |
| 2. Receiving | Fresh/frozen fruit | B – Enteric pathogens B – Patulin in applie juice | Refer to 6.3.3 Intended Consumer and Product Requirements | None | No |
| 3. Storing | All food and product contact raw materials | B – Enteric pathogens | Hazards from poor storage methods and unclear environment. | Yes – The NP L3 includes a raw materials standard and procedure. See Section 2.2.2 Storage of Raw Materials. | No |
| 4. Make | Fresh/frozen fruit | B – Enteric pathogens B – Patulin in applie juice | Hazards carried from previous step | None |  |
|  | Spices | None |  |  |  |
|  | Clean water | None |  |  |  |
|  | Filter / Fining agent | C – Allergens | Refer to 6.3.3 Intended Consumer and Product Requirements | No |  |
| 5. Carbonation | Carbon dioxide | None | Refer to 6.3.3 Intended Consumer and Product Requirements |  |  |

| ###### Process | Inputs | Hazard reasonably likely to occur on or in the product at this step | Justification | Q1. Is there a controlmeasure(s) for the hazards at this step? If yes, identify the control measure1 and then answer Q2 | Q2. Is this step a CCP? |
| --- | --- | --- | --- | --- | --- |
| 8. Packaging | Glass bottles | P – Glass framents | Incorrect filler operation can results in chipping | Yes – correct equipment set-up, equipment maintenance, routine observation during filling, filtration of any affected beverage. | No |
| 9. Storing | Packaged beverage | None |  |  |  |
| 10. Dispatch and Transportation | Packaged beverage | None |  |  |  |

- The procedures for the contorl measures are required to be documented in the National Programme Level 3.